SH 3/Galveston Rd Bridge over Sims Bayou - Shared-Use Path

Project Readiness

Advanced Funding Agreement (AFA) will be needed for this project since the Houston Parks Board (HPB) will provide the 20% match required.

No permits from the railroad are expected to be required before the project can be constructed.

No documentation is needed from the State Historic Preservation Office (SHPO).

TxDOT’s Houston District Environmental Section has reviewed the project and indicated that the criteria for FHWA Categorical Exclusion Action (c)(3) Construction of bicycle and pedestrian lanes, paths, and facilities would be appropriate for this project, a United States Coast Guard (USCG) Permit will be needed as the ped/bike bridge will cross SIMS Bayou.

This project does not require additional acquisition of right-of-way (ROW), no ROW acquisition and/or easements is needed. ROW is owned by City of Houston, which is a project partner.

The existing load bearing capacity for the SH3/Galveston Bridge NBI No. 12-102-B261-05-003 will share with the proposed widened substructure the additional concrete dead load.

Bridge design consideration:

10ft shared use path w/ 1% roadway slope and 2% sidewalk slope.

14 ft total width. 3% combined slope.

60 ft proposed bridge width

298 ft existing bridge length

TxDOT’S Bridge Section provided the preliminary typical section for the proposed shared use path bridge and a detailed construction budget.

This project leverages the HPB Sims Bayou Greenway improvements that connect the share use path to an existing sidewalk along Galveston Rd.

Scoping meeting conducted by TxDOT

TxDOT held several internal meeting to discuss the project.

- 8/30/2018 - Meeting with TxDOT Directors: Director of Consultant Contracts, Director TP&D, Senior Planner on behalf of Dir. of Adv. Transportation Planning, Dir. of Project Development, Director of Design.

  - Directors meeting to discuss improvements with TxDOT Management and get feedback on any preliminary design concerns.

- 5/9/18 & 10/15/18 - Meeting with Houston Parks Board

  - Meeting to discuss improvements, as well as request letters of support.

- 7/25/208 - TxDOT – City of Houston meeting

  - Meeting to discuss improvements, get preliminary feedback, and discuss possibility of City of Houston Public Works and Engineering Department willingness to serve as a project sponsor, if needed.