MKT Trail to White Oak Bayou Connector via Rutland Detention Basin

Summary of Project Scoping

This project does not require additional acquisition of right-of-way. The majority of the project is on TxDOT’s ROW, with a few locations extending into City of Houston and Harris County Flood Control ROW in order to connect to existing facilities. ROW maps are provided as part of the application package.

The northernmost portion of the proposed trail would connect to the MKT Trail (near W. 7th Street) via a culvert with no expected adverse hydrological impacts since there is no water body or constant water presence that could be construed as a wetland.

The northern part of this trail would run through TxDOT owned property adjacent to the Rutland detention pond. The trail would utilize the east berms of the detention pond, which is approximately 15 to 20 ft wide. After the trail navigates around the detention pond east berms, just south of W. 5th St, the trail would then traverse across a portion of detention pond as well as White Oak Bayou via a proposed steel truss bridge where it would then tie into the existing White Oak Trail that runs along the southern side of White Oak Bayou. The steel truss bridge to connect to White Oak Bayou is Alternative A.

Alternatives to the trail alignment and bridge were considered. Alternative B, this included taking the path around a White Oak Bayou outfall area just south of W. 5th St and then continuing the path along the eastern side of White Oak Bayou, traveling under the IH-10 Westbound Frontage Rd Bridge, then up the embankment between the frontage road and the mainlane bridge to the street level at Yale St.  The path would then run alongside Yale St crossing the intersection with the eastbound frontage road, then crossing Yale St in order to access the connection to the White Oak Trail located on the east side of Yale St south of IH-10.

To determine the feasibility of this Alternative B, higher resolution topographical maps or a request for Vegetative Clearing along HCFCD ROW.  Additional information would be needed to determine the width of the flat berm area there along the ROW of Flood Control’s property (between the property line and the drop-off down to the outfall.  If the berm area around the outfall was too narrow and included a drop-off area, it would require a bridge.  The route would also require a new path constructed on the east side of White Oak Bayou (south of the outfall) which in turn would require retaining walls due to the steepness of the existing slope.  The route would also require users to cross intersections in 2 movements to get to the White Oak Trail.

In conclusion, Alternative A is chosen, the trail alignment with the steel truss bridge to connect to White Oak Bayou, since it is a much shorter path and direct connection to the White Oak Trail, that leverages existing ped/bike infrastructure on the south bank of White Oak Bayou and reduces ped/bike exposure to vehicular movements by eliminating street level crossings at I 10/Yale intersections.

In addition to the proposed trail connection from the MKT Trail to the White Oak Trail, existing trail gaps along I-10 EB frontage road would be addressed to ensure connectivity in the ped/bike network from Yale to Heights Blvd.

Project Readiness

A preliminary schematic that shows available ROW and improvements is provided as part of the application package. The TxDOT West Harris Area Office Design Section provided the preliminary schematic as well as the detailed construction budget which is also included in the application package.

No Advanced Funding Agreement (AFA) will be needed for this project since TxDOT will provide the 20% match, which reduces the project timeline by 6-12 months for the AFA process.

No documentation is needed from the State Historic Preservation Office (SHPO).

TxDOT’s Houston District Environmental Section has reviewed the project and indicated that the criteria for FHWA Categorical Exclusion Action (c)(3) Construction of bicycle and pedestrian lanes, paths, and facilities would be appropriate for this project, a U.S. Army Corps of Engineers Permit will be needed as the ped/bike bridge will cross White Oak Bayou.

Due to extensive environmental clearance documentation provided by TxDOT for “IH 10 at White Oak Bayou Flood Mitigation Ponds” (CSJ: 0271-07-292), which included the Rutland’s Detention Pond and nearby Basins (see A7, A9, A10 polygons below), TxDOT’s Houston District Environmental Section indicated that the environmental process may be shorter due to the availability of data and previous thorough analysis.

Scoping meeting conducted by TxDOT

TxDOT held several internal meeting to discuss the project.

- 8/30/2018 - Meeting with TxDOT Directors: Director of Consultant Contracts, Director TP&D, Senior Planner on behalf of Dir. of Adv. Transportation Planning, Dir. of Project Development, Director of Design.

  - Directors meeting to discuss improvements with TxDOT Management and get feedback on any preliminary design concerns.

- 8/31/2018 - Field Visit

  - Field visit to gauge feasibility of proposed improvements, utility conflicts, and as input for the development of a detailed construction estimate and preliminary schematic.

- 9/4/18 - Meeting with key community stakeholder representatives

  - Community stakeholders meeting to discuss improvements get preliminary feedback on community concerns and tap into local knowledge, as well as request letters of support.

- 7/25/208 - TxDOT – City of Houston meeting

  - Meeting to discuss improvements, get preliminary feedback, and discuss possibility of City of Houston Public Works and Engineering Department willingness to serve as a project sponsor, if needed.

Previous public involvement activities related to the proposed project.

- The Heights super-neighborhood council, Oct. 9, 2018.

- Site visit by representatives from Greater Heights and Lazybrook/Timbergrove SNs on July 13, 2018

- There was a site visit by Washington Ave./Memorial Park SN representative today, Oct. 22, 2018.

- Site Visit SN representatives and TxDOT Aug. 31, 2018.