NASA RD 1 Commuter Route to JSC

Summary of Project Scoping

This project does not require additional acquisition of right-of-way. The project is on TxDOT and Harris County ROW. ROW maps are provided as part of the application package. The project proposes to construct improvements to both pedestrian and bicycle facilities along NASA 1 from Space Center Blvd to Kirby Rd.  A proposed shared use path will be constructed along Space Center Blvd from Linkage Rd to NASA 1 within Harris County ROW. From NASA 1 at Space Center Blvd the proposed path will continue as a shared use path northeast along westbound NASA 1 in TxDOT ROW until the ROW narrows just before the southwestern end of the Mud Lake Bridge. The shared use path will then exit onto the roadway as a dedicated on-street bike lane until the available ROW begins to widen out. Once the available ROW widens out the existing Mud Lake Bridge, as part of this project, will be widened enough to provide a 10 ft wide shared use path. At that point, the bike lane will exit the roadway and join with the sidewalk as a shared use path across the bridge where pedestrian and cyclists will be behind a crash-worthy concrete barrier. Near the northeastern end of the Mud Lake Bridge, the ROW begins to narrow again and so the bike path will again exit onto the roadway as a dedicated bike lane for the remainder of the project to connect to the existing bike lane that continues past Kirby Rd.  In addition to the proposed main shared use path, an additional shared use path will be installed within Precinct 2 Clear Lake Park which will tie into TxDOT’s NASA 1 facility at the northeastern end of the Mud Lake Bridge and also near the southwestern corner of Forest Lake Dr.  In addition to these proposed shared use path improvements, this project also proposes to reconfigure the vehicular lanes and relocate the barrier on the eastbound side of NASA 1 as a vehicular calming measure and in order to widen the existing sidewalk on the south portion of the bridge from 6 ft to 9 ft.  The proposed 9 ft sidewalk will also be separated from vehicular traffic by a crash-worthy concrete barrier. Finally, the project also proposes to remove the existing 4 ft sidewalk located behind the curb along NASA 1 (non-bridge locations) from Space Center Blvd to Kirby Rd and replace it with an ADA compliant 6 ft sidewalk and ADA ramps on both sides of NASA 1.

Project Readiness

Based on extensive coordination with TxDOT’S Bridge Section, the TxDOT West Harris Area Office Design Section provided the detailed construction budget which is also included in the application package.  West Harris Area Office Design Section has included in the application package as-built plans with mark-ups and included a couple of bridge cross-sections to help illustrate proposed improvements with TxDOT’s ROW. A preliminary schematic for the portion of the proposed path within Harris County ROW has also been provided by the Harris County Engineering Department and is included in this application package.

Advanced Funding Agreement (AFA) will be needed for this project since TxDOT and Precinct 2 will jointly provide the 20% match required.

No documentation is needed from the State Historic Preservation Office (SHPO).

TxDOT’s Houston District Environmental Section has reviewed the project and indicated that the criteria for FHWA Categorical Exclusion Action (c)(3) Construction of bicycle and pedestrian lanes, paths, and facilities would be appropriate for this project, a United States Coast Guard (USCG) Permit will be needed as the ped/bike bridge will cross Mud Lake.

Scoping meeting

5/14/2018 – Scoping Meeting with Pct 2 and TxDOT.

Site Visit Pct 2 and TxDOT.

5/17/18 – Scoping Meeting with Pct 2 and TxDOT.

Discussed STRAVA data around NASA, Discussed TIP Call for Projects and possible improvements and design concerns.

6/8/18 – Scoping Meeting with Pct 2, TxDOT and NASA employees.

Discussed ped/bike needs around NASA, Discussed TIP Call for Projects and Pct 2 Bay Area Hike and Bike Master Plan.

9/06/18 – Scoping Meeting with NASA, Pct 2, HCED, RPS at NASA.

Discussed efforts on and around NASA property, NASA wants and needs, and how to tie into NASA facilities on E Linkage Rd.

9/11/18 – Scoping Meeting with Pct 2, HCED, RPS at Clear Lake Park.

Discussed efforts on Clear Lake Park property, how to tie into existing sidewalk along NASA Rd 1, and general path through the park.

9/25/18 – Scoping Meeting with TxDOT, Pct 2, HCED, RPS at TxDOT Houston District Office.

Discussed TIP Application coordination, TxDOT plans for NASA Rd 1 improvements, and some preliminary design concerns.