Eastwood Intermodal Terminal Benefits

The following summarizes the BCA based on the attached Eastwood Intermodal Terminal and Associated Pedestrian Transit Corridor Improvements Advance Plan (Plan). This BCA was completed prior to the 2018 TIP Call and was used for advance planning purposes.

| Project | Eastwood Intermodal Terminal Planning Horizon = 2018 to 2038 |  |  |
| --- | --- | --- | --- |
| Parking and Transit Demand | Providing parking spaces for transit users is the priority because of the facility’s primary function as a light rail terminal. In this analysis, the parking demand for transit use was estimated using methodologies outlined in the Plan. Combining the parking demand for both home-based work trips (~65% of demand) and home based non-work trips (~35% of demand), the transit parking demand of the proposed parking facility is estimated to be about 500 in 2018. Currently, Magnolia Park Transit Center has 56 parking spaces available. Subtracting the existing parking supply, the demand of parking spaces would be about 450 in 2018. However, the design of the facility consists of 250 parking spaces; therefore about 164 parking spaces will be used for home-base work trips (65%) and 86 parking spaces will be used for non-work trips; it is estimated that 50% of the non-work trip spaces will turn over once. Given these assumptions, the projected increase of transit ridership is estimated to be ~223,600. |  |  |
| Vehicle Miles Traveled (VMT) Reduction | The proposed parking facilities would reduce auto trips, reduce vehicle miles traveled (VMT) and increase transit ridership. Annual ridership and VMT projections are based on the daily transit demand estimated in the previously completed demand analysis. Assuming an average of annual 234 days for home-based work (HBW) trips (excluding sick leave and vacation leave) and an average annual 312.5 day for home-based non-work (HBNW) trips, it is expected that the proposed parking facility would reduce ~89,500 one way auto trips in 2018 with total annual reduction of VMT of 870,000. |  |  |
| Safety | The project will allow patrons to use transit and as a result 870,000 annual VMT will be reduced within the transportation network. Using the a VMT per injury caused by auto crashes with Houston, there is anticipated to be an annual reduction in about 2.5 crashes and 18.5 injuries within the local transportation network. The estimated 20-year SAFETY benefit of the proposed improvements are: |  |  |
|  | $2018 - $6,800,000 | 7% Discounted Rate - $3,400,000 |  |
| Environmental | Improving park and ride will encourage transit trips instead of using an automobile. The proposed improvement will on average reduce 870,000 VMT throughout the planning horizon. By reducing vehicles trips and cold starts, the annual reduction of harmful air pollutants is approximately 340 tons. The estimated 20-year ENVIRONMENTAL benefit of the proposed project are: |  |  |
|  | $2018 - $470,000 | 7% Discounted Rate - $218,000 |  |
| Economic | Users will save both parking and travel costs incurred by operating a personal automobile. The average parking costs in the CBD and TMC is about $10.00 per day (netting out the $5.00 at the parking facility). The average annual savings in parking to users is about $475,000 million. Additionally, operating a personal vehicle costs about 0.54 per mile. After netting out the fares users would pay, the average annual savings in travel costs to users is about $190,000. The estimated 20-year ECONOMIC BENEFIT of the proposed improvements are: |  |  |
|  | $2018 - $13,300,000 | 7% Discounted Rate - $6,600,000 |  |
| Fare Box | It is expected that the proposed parking facility would increase METRORail ridership by allowing residents within the market area of the proposed facility to park their vehicles at the proposed site and use METRO rail service to travel to destinations. Fare box revenue from increased ridership would increase the fare coverage ratio of METRO’s rail operating expense. Fare box revenue is estimated using $1.25 fare for METRO rail multiplied by the projected additional ridership; 223,600. The estimated 20-year FARE BOX RECOVERY of the proposed improvements are: |  |  |
|  | $2018 - $5,600,000 | 7% Discounted Rate - $2,700,000 |  |
| Residual Life | In 2040, the end of the planning horizon, the improvement will be 20 years old and will have a remaining residual life of 60 percent. It is assumed the that the original value of the assets has depreciated in a linear manner. Therefore, the remaining value of the asset is a benefit beyond the planning horizon. The estimated RESIDUAL LIFE benefit of the proposed improvements are: |  |  |
|  | $2018 - $3,000,000 |  | 7% Discounted Rate - $774,000 |
| Access | The H-GAC uses primary and secondary indicators to determine if a population is an “environmental justice” (EJ) population as defined by Executive Order 12898 and regional policy. The goal of an EJ assessment is to ensure projects are not disproportionately effecting EJ populations. However, transportation projects can also have beneficial impacts to EJ populations; such as providing increased access to recreational and educational opportunities, as well as labor markets. Based on the criteria in Table 1 below, the project area is classified as an EJ area. |  |  |
| Connectivity | The new terminal would be strategically located next to METRO’s Lockwood/Eastwood light rail station. Improved connectivity to transit service will provide park and ride service to transit users traveling to the central business district, Texas Medical Center, and other employment centers, and will contribute to the reduction of vehicles in these congested areas. |  |  |

Eastwood Intermodal Terminal Demographics

| TABLE 1: COMMON MOBILITY ACCESS FACTORS |  |  |  |
| --- | --- | --- | --- |
| Measure | Project Area Block Group(s) | Houston-Galveston Area Regional Averages | Above/Below Regional Average |
| % of Minorities | 95.8% | 62.4% | Above |
| % of Households Below Poverty | 31.2% | 13.7% | Above |
| % of Senior Population (Ages 65+) | 14.8% | 9.8% | Above |
| % of Limited Education Attainment | 56.4% | 17.7% | Above |
| % of Households without Automobile | 17.4% | 5.6% | Above |
| % of Limited English Proficiency | 31.1% | 9.5% | Above |
| Total Factors Above the Regional Average |  |  | 6 |

United States Census Bureau. “2012-2016 American Community Survey 5-Year Estimates” Table B03002. Retrieved in December 2017.

United States Census Bureau. “2012-2016 American Community Survey 5-Year Estimates” Table B17017. Retrieved in December 2017.

United States Census Bureau. “2012-2016 American Community Survey 5-Year Estimates” Table B01003. Retrieved in December 2017.

United States Census Bureau. “2012-2016 American Community Survey 5-Year Estimates” Table B15003. Retrieved in December 2017.

United States Census Bureau. “2012-2016 American Community Survey 5-Year Estimates” Table B25044. Retrieved in December 2017.

United States Census Bureau. “2012-2016 American Community Survey 5-Year Estimates” Table B16004. Retrieved in December 2017.