Safety benefits

Included are the following documents to support the additional Safety Benefits:

- Model Data spreadsheet- prepared by H-GAC for Friendswood Lakes Blvd. – FM 528 to FM 2351; and, FM 518 – FM 528 to FM 2351 for parallel roadway impacts; we have added Traffic Study date based on the included Traffic Study by City of Friendswood and GC Engineering, Inc. dated May 4, 2018.

- Scoring Criteria spreadsheet- Displaying the three data sets:

Friendswood Lakes Blvd.- FM 528 to FM 2351

FM 518- FM 528 to FM 2351

Traffic Study Friendswood Lakes Blvd.- West Blvd. to FM 528

- Form -Safety

Friendswood Lakes Blvd.- FM 528 to FM 2351

- Form- Safety

FM 518- FM 528 to FM 2351

- Form- Safety

Traffic Study Friendswood Lakes Blvd.- West Blvd. to FM 528

- The Traffic Impact Study looked at the immediately adjacent side of Friendswood Lakes Blvd. at FM 528 and two other intersections. GCE’s Subcontractor, C.J. Hensch & Associates, Inc., collected 24-hour turning movement counts (TMC’s) at three screen locations along the study corridor on March 22, 2018. The study did not consider the broader growth that will lead to added traffic as League city Parkway is connected through Friendswood to Pearland Parkway. Existing 24-hour traffic counts were obtained at the three intersections indicated below:

- FM 528 at Friendswood Lakes Blvd.

- FM 528 at Falcon Ridge Blvd.

- FM 518 at West Blvd.

The LOS for FM 528 at Falcon Ridge Blvd intersection can be improved due to the proposed Friendswood Lakes Blvd extension to West Blvd. The Report evaluated the existing traffic conditions on major streets in the proposed project. The major streets include FM 528, Falcon Ridge Blvd., FM 518, and West Boulevard. The report was to

- Identify possible future developments along Friendswood Lakes Blvd.

- Develop possible traffic distribution along proposed project, and

- Evaluate if the Friendswood Lakes Blvd. extension is needed to maintain safe operation

and acceptable level of service on the roadway system.

The focus was to calculate the volume of traffic that the future developments in the project area would generate, and to assess the impact of that traffic and the traffic distribution on the proposed Friendswood Lakes Blvd. extension. This project was a high priority for the community and was approved in the 2017 Galveston County Bond election. This was a critical step in the connection from League City Parkway to Pearland Parkway.