Shepherd Durham Daily Delay Calculations

For this roadway project, intersection delay by approach for AM peak hour and PM peak hour was available based on a Traffic Analysis Report completed by Jones and Carter. In addition, hourly traffic volumes along Shepherd Drive and along Durham Drive at one location were available.

To calculate total daily delay was calculated by first calculating the total peak hour delay (vehicle – hours) at each of the four intersections analyzed (Shepherd Drive at 20th Street and at 11th Street, Durham Drive at 20th Street and at 11th Street).  To calculate daily delay, it was assumed that hourly volumes along the cross streets follow approximately the same patterns as the traffic along Shepherd and Durham, thus further assuming that hourly delays calculated for peak hours will apply proportionally to other hours of the day for hours that have at least 70% of the peak hour traffic volumes.

Table 1. Daily Delay (veh-Hours) for Shepherd Dr

| Time of Day | Hourly Volumes along Shepherd Dr | % of AM Peak | % of PM Peak | Delay (Veh-seconds) at Intersection of Shepherd Dr at |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 20th St | 11th St |
| 7:00 AM | 970 | 1 |  | -5689 | 79263 |
| 8:00 AM | 1000 | 1.03 |  | -5864.95 | 81714 |
| 9:00 AM | 966 | 0.99 |  | -5665.54 | 78936 |
| 10:00 AM | 915 | 0.94 |  | -5366.43 | 74769 |
| 11:00 AM | 1173 | 1.21 |  | -6879.58 | 95851 |
| Noon | 1275 | 1.31 |  | -7477.81 | 104186 |
| 1:00PM | 1298 | 1.34 |  | -7612.7 | 106065 |
| 2:00PM | 1376 | 1.42 |  | -8070.17 | 112439 |
| 3:00PM | 1754 |  | 0.76 | -27472 | 94203 |
| 4:00PM | 2111 |  | 0.91 | -33063.5 | 113376 |
| 5:00PM | 2314 |  | 1 | -36243 | 124279 |
| 6:00PM | 2046 |  | 0.88 | -32045.5 | 109885 |
| Total Daily Delay (Veh-hours) |  |  |  | -50.4 | 326.4 |

Table 2. Daily Delay (veh-Hours) for Durham Drive

| Time of Day | Hourly Volumes along Durham Dr | % of AM Peak | % of PM Peak | Delay (Veh-seconds) at Intersection of Durham Dr at |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 20th St | 11th St |
| 7:00 AM | 2409 | 1 |  | -24898 | 164996 |
| 8:00 AM | 2264 | 0.94 |  | -23399 | 155064.7 |
| 9:00 AM | 2064 | 0.86 |  | -21332 | 141366.4 |
| 10:00 AM | 1837 | 0.76 |  | -18986 | 125818.9 |
| 11:00 AM | 1908 | 0.79 |  | -19720 | 130681.8 |
| Noon | 1835 | 0.76 |  | -18965 | 125681.9 |
| 1:00PM | 1742 | 0.72 |  | -18004 | 119312.2 |
| 2:00PM | 1677 | 0.70 |  | -17332 | 114860.2 |
| 3:00PM | 1689 |  | 0.88 | -143.9 | 522.68 |
| 4:00PM | 1722 |  | 0.90 | -146.7 | 532.89 |
| 5:00PM | 1913 |  | 1 | -163 | 592 |
| 6:00PM | 1751 |  | 0.92 | -149.2 | 541.87 |
| Total Daily Delay (veh-hours) |  |  |  | -45.3 | 300.0 |